4006BIO: Human Physiology: Homeostasis and Health

Physiology Lab 4: ECG

- Record an ECG and identify the major components.

- Explain the temporal relationship between the ECG and the finger pulse.

- Interpret events in the cardiac cycle, and relate these to features in the ECG and the finger pulse.

- Show the electrode placement to perform an 12-lead ECG.

- Describe how the different "views" of the heart are provided by the 12 leads.

Experiment 1A: Equipment Setup

Setup

- Make sure the PowerLab is connected to your computer and turned on.

- Connect the electrode cables to EARTH, CH1 NEG, and CH1 POS on the Bio Amp cable.

- Connect the Bio Amp cable to the Bio Amp input on the PowerLab.

- Connect the finger pulse transducer to Input 1 on the PowerLab.

Attaching ECG electrodes

Procedure

- Remove any jewelry from the volunteer's wrists and ankles.

- Use a pen to mark a small cross on the volunteer's skin where each electrode will be placed (that is, both wrists and the right leg).

- Lightly abrade the skin with abrasive gel, and clean the area with alcohol swabs.

- Snap disposable electrodes onto the three electrode cables.

- Attach the electrodes to the following prepared areas on the volunteer's skin:

- Place the positive electrode on the left wrist.

- Place the negative electrode on the right wrist.

- Place the earth electrode on the right leg.

- Ensure the finger pulse transducer is attached to a distal part of the volunteer's middle or index finger.

Experiment 1A: ECG and Pulse: At Rest

Procedures

The volunteer should relax and sit as still as possible to minimize signal artifacts due to movements.

- Enter the volunteer's name into the Comment panel.

- Select Start to record, then select Add.

- Use Auto Scale to see the data clearly in all channels.

- After 60 s, select Stop. Leave the ECG cables and electrodes attached to the volunteer.

- Examine the ECG recording. Compare it to a model ECG recording.

Results

Experiment 1A: ECG and Pulse: With Exercise

Procedures

! Using the same procedure as you did for 'rest' now, using the same trace, complete 2 minutes exercise (or enough to significantly raise your heart rate) by doing some star jumps or simply jogging on the spot and then reconnect the electrodes and obtain another ECG.

- Select Start to record, then   Enter the comment - 'After 2 minutes exercise', by pressing Add.

- Use Auto Scale to see the data clearly in all channels.

- After 60 s, select Stop. Leave the ECG cables and electrodes attached to the volunteer.

- Examine the ECG recording. Compare it to your resting trace.

Results